# Cable Harness Design Tool ? Updated Final Specification

# 1. Problem Statement

Designing point-to-point cable harnesses often requires engineers to repeatedly consult static wire sizing charts, part-number references, and packaging tables. These steps are rule-based and deterministic, but when performed manually they are time-consuming, repetitive, and prone to inconsistency. This project addresses that problem by automating core cable design decisions so that user-defined electrical inputs can be converted into structured engineering outputs more efficiently and consistently.

# 2. Objective

The objective of this project is to develop a prototype cable harness design assistant that automates standard point-to-point cable design tasks within a constrained MVP scope. The tool should accept conductor-level electrical inputs, select wire gauge using reference data, map part numbers and attributes, generate a wire list and bill of materials (BOM), estimate packaging recommendations, and present the results in both a terminal workflow and a polished Streamlit interface.

# 3. Design Principles

Simplicity over completeness: prioritize a working and understandable MVP over full engineering fidelity.

Deterministic outputs: identical inputs must return identical results.

Automation of repeatable processes: focus on replacing manual table lookups and spreadsheet-style decision steps.

End-to-end usability: support a complete workflow from input through output, export, and demonstration.

Extensibility: keep the architecture modular so future enhancements can be added without rewriting the system.

# 4. Final Prototype Scope

In Scope

Point-to-point cable and harness design only.

User-defined inputs: number of conductors, signal names, current values, cable length, and optional wire color overrides.

Reference-data-driven AWG selection using local ampacity values.

Attribute mapping for wire color and wire part numbers.

Length-aware engineering guidance, including a simple AWG upsizing rule for longer cable runs.

Voltage-drop review flags and per-wire engineering notes.

Bundle diameter estimation and sleeve recommendation using extracted reference tables.

Wire list, BOM, packaging summary, CSV export, and scenario-test report generation.

Streamlit GUI with harness routing visualization and downloadable outputs.

Out of Scope / Deferred

Temperature derating calculations.

Exact voltage-drop calculation beyond the current warning logic.

Complex multi-branch harness topologies.

CAD-accurate schematic or drawing generation.

External databases or live part integrations.

Automated Excel ingestion as an end-user workflow.

# 5. Inputs

Primary user inputs include: number of conductors (integer), signal name per conductor (string), current per conductor (float, amps), cable length (float, feet), and optional user-selected wire color.

# 6. System Logic

# 6.1 AWG Selection

AWG selection is driven by shared reference data rather than a fixed hardcoded five-row lookup. The current prototype supports 22, 20, 18, 16, 14, 12, 10, 8, 6, 4, and 2 AWG. Unsupported current values above the prototype range are rejected with a clear error.

# 6.2 Length-Aware Sizing

If cable length exceeds 15 ft, the tool recommends one larger conductor size when available to reduce voltage-drop risk. This adjustment is surfaced as an engineering note rather than hidden logic.

# 6.3 Attribute Mapping

Selected AWG values are mapped to wire attributes using internal lookup rules. The prototype maps AWG and color to wire part numbers in a deterministic format, while also allowing a user-selected color override from the supported set.

# 6.4 Voltage Guidance

The prototype does not perform a full voltage-drop calculation for sizing decisions, but it provides review flags based on cable length thresholds and includes engineering notes when longer runs or high load ratios deserve attention.

# 6.5 Packaging Logic

The tool estimates bundle diameter using RMS wire diameter and bundle factor reference data, then selects a sleeve from fit-range lookup data when possible. Sleeve length is calculated with 3 percent margin, 6 in tolerance, and rounding to the nearest 0.5 ft. If no sleeve range fits the bundle diameter, the tool preserves the wire design output and surfaces a packaging warning instead of failing.

# 7. Outputs

# 7.1 Wire List

The wire list includes signal name, current, AWG, color, wire part number, shared length, and an engineering note field.

# 7.2 Bill of Materials (BOM)

The BOM aggregates wire quantities by total required length and includes sleeve material when a valid sleeve fit is found. BOM data is available both on-screen and through CSV export.

# 7.3 Packaging Summary

Packaging output includes bundle diameter, sleeve recommendation, sleeve fit information when applicable, calculated sleeve quantity, and any packaging warning.

# 7.4 Harness Routing Visualization

The Streamlit interface includes a simplified harness routing view for demonstration. It is not CAD-accurate, but it shows conductors, labels, length, and packaging context in a readable graphical form.

# 7.5 Exportable Results

The prototype exports wire list and BOM data as CSV files. It also supports scenario-based testing logs and HTML report generation for demonstration and reporting purposes.

# 8. System Architecture

The final system consists of the following modules: Input handling in main.py and streamlit_app.py, AWG selection logic in modules/awg.py, signal and wire-list mapping in modules/mapper.py, BOM/CSV formatting in modules/output.py, and packaging calculations in modules/packaging.py. Reference data is stored in simplified CSV tables under reference_data.

# 9. Development Process Followed

The project was developed iteratively in phases. Implementation began with core AWG selection and signal processing, then expanded into wire-list and BOM generation, followed by UI and visualization improvements, engineering-rule refinement, packaging features, and finally automated testing and report generation. Scope was actively managed during development; temperature derating and more complex harness modeling were intentionally deferred to preserve a reliable MVP.

# 10. Acceptance Criteria Used For The Prototype

The user can define multiple conductors and associated current values.

The system assigns AWG values consistently from reference data.

Wire attributes and part numbers are mapped deterministically.

The tool generates a readable wire list and BOM.

The tool surfaces voltage and packaging guidance without failing normal design cases.

The GUI supports a usable end-to-end demonstration workflow.

Core functionality is validated with unit tests and scenario-based tests.

# 11. Validation Approach

Validation was performed using both unit-style automated tests and scenario-based test cases. Unit tests covered AWG selection, wire-list generation, note generation, BOM logic, and packaging edge cases. Scenario tests exercised realistic end-to-end harness cases, including a no-fit sleeve case, and generated TXT, CSV, JSON, and HTML logs for documentation.

# 12. Future Development

Potential future work includes more explicit voltage-drop calculations, deeper packaging traceability, expanded packaging material support, broader engineering rule coverage, and more advanced harness modeling beyond the current point-to-point prototype scope.